Fig. 1. Nutidskort. Er med

Figur S1. Chlorophyll and Secchi-depths in Lake Arresø over time. Mean summer values and range.

Tabel S1. Mean chemical composition with depth in sediment (organisk stof, calcium, Fe and Fe:TP ratio).

Tabel 1. Basal karakteristik. Catchment area  215 km2; Cultivated area  116 km2; Lake area 39,9 km2, Mean depth 3,0 m (max depth 5,9 m), Water retention time 2,4 year (1,3-5,9, median 3,1 years), Alkalinitet. Shallow lakes

Tabel 2. Mean annual input of TP and TN to Lake Arresø and outlets from sewage treatment plants in 1970s, 1985-1987 and 2018-2021. Mean annual input to Lake Arresø in the 1970s was 82 tons TP and 508 tons TN. Outlets during storm flow events in the two main cities are included in the sewage outlets in 2028-2021.

Fig 8. Summer concentrations of TP, TN, TN: TP ratio and progressive increase of TP during summer in Lake Arresø over many years (xx-xx). TP, TN and TN:TP ratio are given as mean values (the line) and range.

Massebalance

Fig. 2. Annual input and output of TP and TN in Lake Arresø during many years (xx-xx).

Fig 3. Annual retention of TP and TN versus water retention time (WRT) in Lake Arresø during many years.

Figur S 1. Årlige værdier af TN input (x) versus TN removal (y) over alle årene. Sikkert på log-log akser.

Figur S2. Annual area specific TN removal in Lake Arresø, Lake Solbjerg and Lake Strødam as a function of area specific TN input.

Fig. 4. Annual removal of TN and TP retention in Lake Strødam and Lake Solbjerg in the years following their construction OK.

Sedimenter

Sedimentkarakteristik

Tabel S3. C/N, N/P and C/P over årene og mod dybden

Figur 5. Sediment TP concentrations (per DW) with depth in different years.

Tabel S4. P-associeret med forskellige stoffer

Figur 6. The decrease of concentrations of TP (per DW), organic carbon and TP (per organic carbon) in surface sediment (0-5 cm depth) over the study period.

Tabel 3. TP-pools in sediment depth strata over the period.

Nutrient dynamics following 50 years of reduced loading in the large, shallow Lake Arresø

Kaj Sand-Jensen1, Jonas Stage Sø2 and Theis Kragh2

1Freshwater Biological Section, Department of Biology, University of Copenhagen, Universitetsparken 4floor, 2100 Copenhagen.

Introduction

Lake Arresø is a shallow, alkaline, hypertrophic lake and the largest in Denmark (40 km2, Andersen 1976). It was a relatively deep freshwater lake when the Weichel Glaciation ended about 11.000 years ago. Subsequently the lake turned into a shallow brackish fjord and then to a plant-rich freshwater lake 4.000-5.000 years ago when the shallow connection to the saline Kattegat was closed by land uplift and coastal formation of sand bars. During the 18-19th.centuries the lake became rich in phytoplankton and lost most submerged vegetation (COWIconsult 1989, Sand-Jensen et al. 2025).

During the 20th.century, Lake Arresø turned hypereutrophic due to high nutrient inputs of domestic sewage from 60-70.000 residents and nutrient losses from cultivated fields in the catchment (Table 1). According to references in Olesen (1976), Lake Arresø already had become very turbid in the 1930s from dense phytoplankton blooms and frequent resuspension of sediment particles due to the low mean water depth relative to the large lake surface area and the high wind exposure (the Osgood index (1988) is only 0.5 according to mean water depth (m): (lake area km2)0.5).

Detailed field measurements revealed that resuspension occurred about half of the time and at most involved the upper 2 cm of unconsolidated sediments of low density homogeneously distributed across the main lake bed of relatively uniform water depth (COWIconsult 1989, Søndergaard et al. 1992). Plans to restore the lake by sediment removal as in Lake Trummen, Sweden (Björk 1985) were heavily disputed in the 1980s (Hovedstadsrådet 1989). However, careful evaluations concluded that costs were too high and chances of success were too low due to the large bottom areas of fine sediments (about 31.6 km2) and the frequent spatial shifts in sedimentation of fine organic matter depending on wind velocity and direction (COWIconsult 1989). Moreover, the short water retention time (mean 2.5 years) means that lake conditions remain highly dependent on the external nutrient sources in the catchment (Kalff 2002).

Many studies of the response of shallow lakes after the reduction of external nutrient sources show substantial net release of sediment nutrients for 10 or more years slowing the improvement of lake ecological qualities (Søndergaard et al. 1993; 1999; Welch & Clarke 2005). Phosphorus (P) retention in winter may be replaced by total phosphorus (TP) release in summer and raise in-lake TP concentrations 2-6 fold (Andersen 1976, Phillips et al. 1994, Søndergaard et al. 1999). Phosphorus may be released from adsorbed P, redox sensitive Fe-P compounds and labile organic-P compounds, while Ca-P, Fe-P minerals and Al-P minerals and certain organic P-compounds are recalcitrant (Welch & Clarke 2005, Søndergaard et al. 2012).

Phosphorus release from aerobic surface sediments is stimulated by organic degradation at increasing temperature, but reduced by high nitrate concentrations elevating redox levels in surface sediments and improving Fe-P binding, while the pH influence is smaller and variable though high summer pH may release P from Al- and Fe(OH)3 (Jensen and Andersen 1992, Jensen et al. 2006). Lakes with high Fe:TP ratios > 15 in surface sediments show a reduced P release during summer possibly due to firm P-binding in Fe-P minerals (Jensen et al. 1992, Immers et al. 2015). With increasing depth in sediments there is a marked increase in the proportions of recalcitrant Fe-P and Ca-P minerals as well as recalcitrant organic P (Ketsev et al. 2006).

Many freshwater researchers in the 1980s suggested that Lake Arresø should offer a long-term internal loading due to gradual loss of large P pools in the sediments accumulated during decades of very high external loading and high primary productivity (Andersen 1976, Olesen 1976, COWIconsult 1989). However, substantial reduction of external phosphorus sources from domestic sewage in the 1980-1990s following the Danish Water Environmental Plan I (1987) rapidly reduced in-lake P and chlorophyll concentrations (Sand-Jensen 2014). Over recent decades, nutrient inputs from domestic sewage have been markedly reduced by closing down more than 30 small sewage treatment plants (Olesen 1976) and restricting main sewage treatment at two newly build and very efficient water treatment plants with combined mechanical, advanced biological and chemical treatment (Sand-Jensen et al. 2025). They presently remove about 98% of TP and 90% of total nitrogen (TN) from the raw sewage. Moreover, to facilitate improvement of Lake Arresø and assist biodiversity, four small new shallow lakes were constructed in the catchment during the 1990s with the goal of retaining nutrients in their sediments and, in particular, reduce TP inputs to Lake Arresø (Sand-Jensen 2003).

Nutrient losses from cultivated field have remained much less regulated, though some legal regulations required nutrient application per surface area reduced below certain thresholds, cultivation of follow-up crops during winter and establishment of uncultivated 10 m wide buffer zones along major streams (Water Environmental Plan II and III, 1998, 2004) may have reduced nutrient losses appreciably. After 2016, however, the 10 m wide buffer zones were reduced to only 2 m and farmers were allowed to increase nutrient application on the fields (Landbrugspakken 2016). As a likely consequence, nation-wide measurements in 153 lakes found significant increases of TP, TN and chlorophyll from 2011-2016 to 2017-2022 (Johansson et al. 2024). The detailed development in Lake Arresø has remained unexplored (Sand-Jensen et al. 2025).

Lake Arresø’s outflow empties into the outer broad in Roskilde Fjord, a saline area with eelgrass and diverse marine macroalgae (Middelboe and Sand-Jensen 2000). The phytoplankton and macroalgal growth in the broad are mainly N-limited according to many bioassay growth experiments (Geertz-Hansen et al. 1993, Pedersen & Borum 1996). The outlet from Lake Arresø is a major nutrient source to Roskilde Fjord and its eutrophication. While the main emphasis has been on the influence of P on ecological quality measures in Lake Arresø, the main catchment effect on Roskilde Fjord is the N input via the outlet from Lake Arresø. Nitrogen outlet from Lake Arresø has been reduced thanks to much better sewage treatment, probably slightly lower outlet from the farmland and loss by denitrification in sediments of Lake Arresø and newly constructed lakes. The combined effect of these efforts are unknown but may appear as a reduced outlet from Lake Arresø.

The long-term development of nutrient pools in sediments during nutrient abatement has been measured in only a few lakes (Søndergaard et al. 1993, Kiani et al. 2020). Instead internal loading by nutrient release from sediments is often calculated from mass balances of nutrient inputs by hydrological inlets and atmospheric deposition and hydrological outlets (Søndergaard et al. 1993, 2003, 2012, Welch & Cooke 2005). The common finding is that shallow lakes experiencing high nutrient inputs for decades have accumulated large sediment pools that gradually release nutrients to lake waters over several years following nutrient abatement, thus markedly slowing the improvement of lake nutrient status. However, the story may be different in large shallow lakes experiencing almost permanent resuspension, whereby lake waters and sediments are in much closer contact and nutrient pools in lake water and surface sediments may work as an entity and decline in concert.

Our main objectives were to evaluate the external nutrient sources and inputs to and nutrient outputs from Lake Arresø, in-lake water nutrient and sediment dynamics as well as water quality during the last 50 years. Specifically, we evaluated: (i) coupling between N and P inputs and outputs, (ii) N and P removal in Lake Arresø and newly constructed lakes in the catchment, (iii) sediment P release during summer (internal loading) and the contribution of sediment P pools to the lasting eutrophication, and (iv) the historic changes in N output to the downstream estuary.

Materials and methods

Lake Arresø’s nutrient dynamic, phytoplankton chlorophyll and macrophyte vegetation were studied by several authors and agencies in the 1970-1980s (Andersen 1976; Hovedstadsrådet 1975, 1982, 1988; Olesen 1976, COWIconsult 1988 and references in them). Early measurements accord with high standard chemical analyses (Strickland and Parsons 1968, Limnological Methods 1973). From the late 1980s to the early 2000s, detailed measurements following a standard program were conducted by the Frederiksborg County and, thereafter, by The Agency of Environmental Protection (Lauritsen et al. xxxx). The measuring program followed the standards of the Danish National Monitoring Program (NOVANA), which has been running since 1989. It includes mid-lake integrated water sampling with depth in the well mixed water column at monthly intervals during winter and bimonthly during summer for the main physical (i.e. temperature and Secchi depth), biological (phytoplankton chlorophyll, composition and biomass) and chemical variables. The chemical analyses included TP and TN  analysed by spectrophotometry after persulphate digestion (Koroleff 1969, Solorzano 1969). Inorganic phosphate, ammonia and nitrate were measured according to several authors (Murphy and Riley 1962, Strickland and Parsons 1968, Solorzano 1969). Total alkalinity was measured by acidimetric titration (Rebsdorf 1973) and chlorophyll initially by acetone extraction on GF/C filters (Golterman 1969) and later replaced by ethanol extraction (Jespersen and Christoffersen 1987).

Sediment cores were sampled from the three main sediment areas outlined by COWIconsult (1988) in transparent Perspex tubes (inner diameter 56 mm) using a Kajak sampler (Kajak et al. 1965). In 1988, four separate sediment cores were analysed from the main sediment areas, but later only single cores were analysed from the main sediment areas. Sediments were sliced into six cm-depth strata (0-2, 2-5, 5-10, 10-20, 20-30 and 30-40 cm), carefully mixed and analysed for wet weight, dry weight (DW at 105 o C), organic dry weight (Org DW as loss on ignition at 550 o C). Organic carbon was estimated as Org DW*0.44 (Ref). Total nitrogen was analysed on dried material as above, while TP and total Fe (Fe) were analysed on the ash content after dissolution and heating in HCl (Andersen 1976) followed by measurements of dissolved P by the molybdate methods (Murphy and Riley 1962) and dissolved Fe by the bipiridyl method (Heaney and Davidson 1977). Calcium content was measured on the ash either as weight loss on combustion at 900 oC or after dissolution in HCl, adjustment to high pH and titration with EDTA (Limnological Methods 1973), The calcite content (CaCO3) is estimated according to thw molecular weight of calcite (100) by multiplying the Ca content (atomic weight 40) by 2.5 assuming the majority of Ca is present as calcite.

Fractionation of P in the calcareous sediments of Lake Arresø was performed in 1988 and 1993 and mostly followed Hieltjes & Lijklema (1980) by stepwise extraction of P in NH4CL (accounting for P loosely adsorbed to organic matter, clay and calcite), extraction in NaOH (accounting for adsorbed P to Al(OH)3 and Fe(OH)3) and extraction in HCl (accounting for Ca-P and Mg-P). TP minus the above fractions is Residual-P and is mainly immobile and refractory organic P. Traditionally exchangeable P is regarded as the sum of adsorbed-P and mobile Fe-P and organic P. However, evaluation of mobile P according to dissolution by different solvents is not straightforward as extraction methods depend on the type of sediment and P-binding depends on pH and redox conditions (Søndergaard et al. 2003).

Mass balance calculations were based on 18-24 annual samplings in the four main inlets and the single outlet from Lake Arresø. Nutrient transport was calculated by multiplying TP and TN concentrations with water discharge (Q). Discharge was calculated every hour and integrated to daily values at permanent hydrometric stations constantly recording water level (H) in the inlet and outlet streams and calibrating the Q-log H relationships over the year by direct measurements of Q using mechanical and electromagnetic current meters. A small part of the catchment (Asserbo region) with sandy soils drains by subsurface flow into Lake Arresø. Its nutrient input was added using TP and TN concentrations from an adjacent similar area with surface flow and the discharge calculated from the actual surface area and validated by the lake’s water balance that accounted for surface inlets and outlets, changes in lake water volume according to continuous lake water levels measured at two hydrometric stations (one to the east and one to the west in the lake basin) as well as precipitation measured at a nearby meteorological station and evaporation calculated from lake water temperature and wind velocity (Hovedstadsrådet 1987, Nova 2003). Atmospheric input was set at 0.1 kg P ha-1 and 15 kg N ha-1 for the entire period according to nationwide measurements (Ellermann et al. 2010).

Results

Land use and Secchi-transparency - Land use in the catchment in 2003 was divided between towns and infra structure (17%), cultivated land (52%), forests (23%), wetlands (5%) and nature areas (3%) (Frederiksborg Amt 2003). Secchi-depth measurements in 1934 and 1957-1958 and more frequent measurements in 1973 showed very low values (about 0.30 m; Andersen 1976, Olesen 1976) while many measurements in the 1980-1990s and further on showed only slightly higher mean values (about 0.40-0.70 m) as a reflection of hypereutrophy, unclear-waters and dense phytoplankton populations in the lake (Fig. 1).

Input and output of P and N – Input of TP and TN as well as output from Lake Arresø declined extensively from the 1970s to the early 2020’ies (Fig. 2). Annual TP input to Lake Arresø decreased from about 82 tons in the 1970s (Andersen 1986) to about 6-7 tons in the 2000-2020s (Table 2). Annual outlets followed the same time course and magnitudes as inlets of TP, but with an annually shifting balance between the two. Outlets mostly exceeded inlets in the 1990s, but in recent years, inlets slightly exceeded outlets, though within the statistical error of differences between inlets and outlets.

Markedly reduced TP inputs from 1973 and onwards were mainly due to reduced input from domestic sewage by transferring treatment from about 30 small water treatment plants to only one small still functioning and two newly modern plants with high purification efficiencies in the two main cities. Comparative data in 1985-87 and 2018-21 showed a drop of annual sewage outlets from 62.6 tons TP and 199 tons TN to only 2.2 tons TP and 23.3 tons TN, while annual inputs to Lake Arresø dropped from 72 to 6.8 tons TP and 449 to 247 tons TN over the same period (Table 2). Thus, annual sewage output of TP, including overflow from buildings, roads and pavements in the cities, decreased to only 3.5% over the period and its relative contribution to total input to Lake Arresø dropped from approximately 87 to 32%. From totally dominating the TP input to Lake Arresø in the 1970-1980s, urban input presently contributes about one-third, while rural input estimated by difference is accountable for the remaining two-thirds (Table 2). In 1988, the nutrient loss from cultivated fields was estimated at 5.5 tons P (Table 3.5.14 in COWIconsult 1988) supporting the assessment that they presently represent the main TP source input to Lake Arresø.

Mean annual TN input to Lake Arresø decreased 2-fold from the 1970s to 2018-21 (Table 2). Over the period 1985-87 to 2018-21, the TN loss from cities dropped to 12% of the initial and their contribution to the total input to Lake Arresø declined from about 44 to just 9%. The loss from open land and cultivated fields in particular dominates the TN input to Lake Arresø.

Retention of P and N in Lake Arresø and new lakes – Annual input and output of TP from Lake Arresø changed almost in concert over the years, though long water retention time retained some TP in the lake, while short water retention time released some TP (Fig. 2). In contrast, TN input to Lake Arresø exceeded TN output via the outlet every year over the 50 years (Fig. 2 and Fig. S1). The annual retention of TN in the lake as a percentage of the input was high and increased with water retention time leaving more time for TN removal within the lake (Fig. 3). Because the nitrogen pool in the upper sediments did not increase, but in fact decreased in surface sediments from 1988 to 2018 along with the reduction of organic matter (see later), this removal primarily represents a net loss by denitrification. Over the years, the removal in magnitude decreased along with the reduced input (Fig. S1).

During the first two years after constructing the shallow Lake Strødam and Lake Sobjerg they removed a high proportion of TP and TN input from the stream Pølå (Fig. 3). The loss of TP during passage was probably due to net particle sedimentation and P-adsorption in the submerged terrestrial soils. After the two initial years, however, TP retention became minimal as a balance between particle sedimentation-resuspension and P-adsorption and release was obtained. The loss of TN may initially be due to a combination of net particle sedimentation (along with TP removal) and denitrification in the sediments. After the two initial years, nitrogen loss continued in Lake Solbjerg likely due to ongoing denitrification.

Assuming that the annual loss per sediment area in the four constructed shallow lakes in the catchment to Lake Arresø was the same as the annual mean loss in Lake Solbjerg at 14.5 g TN m-2 in year 3, 4 and 5 after its construction, the combined annual loss was 21.5 ton N from the four lakes (total surface area 147.8 ha). The mean annual N loss by passage of Lake Arresø in the recent 10 years 2012-2021 was 95.4 ton N. Thus, total removal was 117 ton N in all five lakes in the catchment. Compared to the mean annual TN output of 84 ton from Lake Arresø and, thereby, from the entire catchment, in the same 10 years, the combined TN removal by denitrification in the five lakes was 39% higher. With no denitrification taking place, TN output from the Lake Arresø catchment to the downstream estuary Roskilde Fjord would have been 2.4-fold higher.

In-lake P and N concentrations – Mean summer TP and TN concentrations (in mg m-3) peaked in 1973 to mid-1980s (app. 0.75 TP and 3.5 TN), decreased to a minimum 2000-2010 (app.  0.15 TP and 1.8 TN) and slightly increased recently (app. 0.20 TP and 2.5 TN, Fig. 8). Thus, recent TP concentrations were about 20% and TN levels about 60% of those in 1970-1980s. When the high early TP-rich input from small sewage treatment plants with a TN/TP ratio of about 3 (Lønholdt 1973) dominated the conditions in Lake Arresø, the in-lake TN/TP ratio was about 5, but with later smaller total inputs from sewage treatment plants and particularly efficient P-purification, in-lake TN/TP ratios increased to 15-20 in recent years (Fig. 8).

During summer TP markedly increased in the water column likely by release from surface sediments due to mineralization of organic matter and release of P desorption at high pH (often 9.5-10) and low oxygen and low redox potential. The increase of TP in the water (in mg m-3) from early to late summer was x-x in 1970-1980, gradually decreasing to a minimum of x-x in 2000-2010 and increasing slightly to xx-xx most recently (Fig. 8). This net release to the water column corresponds to a total TP pools in the lake of xx-xx tons in 1970-1980 and xx-xx tons in 2000-2010.

Sediment composition and dynamics – Lake Arresø’s sediments were sampled in November-December and studied in six sediment strata down to 40 cm depth (Table Sx). Sediment cores were retrieved from spatially homogenous sediment areas covering 31.6 km2 of the lake (COWIconsult 1988). The remaining sediment areas represent hard marine sediments that likely have no important nutrient exchange with the water. The examined sediment strata are rich in carbonates (min. 32-41% DW) and organic matter (22-38% DW), but poor in Fe (0.7-1.4%). The mean Fe:TP ratio was 7.2 (range: 6.8-13.7) at the very surface increasing to 20.8 at 30-40 cm sediment depth.

Sediments were modestly rich in TP and rich in organic matter in surface sediments, but variability of duplicate-triplicate measurements and spatial variability resulted in changes over the full 40 cm-depth range between years that were neither systematic nor significant over the 30 years of measurements (Fig. 9).

The TP and organic content followed each other closely and decreased with sediment depth, but the change from 0-2 cm to 2-5 cm was small and not significant and the decrease to the 5-10 cm stratum was only about 10%. Averaged over the entire period, TP decreased to 42% at 30-40 cm relative to the surface sediment at 30-40 cm in the sediment and mean organic content decreased to 60% (Ref). The mean C:P-ratios were within a narrow range (94-99) in the upper 10 cm of the sediments and increased to 149 at 30-40 cm depth due the steeper decline of TP than organic content (Table S4).  The C:N-ratios ranged between 6.8 and 10.0 independent of sediment depth, while N:P rations ranged between 6.8 and 16.0.

Calculations of pools of TP and organic content with sediment depth require knowledge of sediment density (i.e. dry mass relative to sediment volume) and as this data was not available, a mean density of 1.1 g cm-3 was used for all calculations. This value is close to those (1.07-1.11 g cm-3) calculated for the 1988 data setting volume density at 2.6 g cm-3 for inorganic matter and 1.05 for organic matter (Søndergaard xxxx).

Measurements of TP and organic matter over the entire period (1988-2018) including all sediment depths down to 40 cm were quite variable and did not reveal systematic and significant temporal changes. Measurements were more reproducible and temporal changes more distinct in the upper 5 cm of sediment (i.e. the strata 0-2 and 3-5 cm) in closer contact with the lake water. The TP and org. C concentrations relative to dry matter in the upper 5 cm as well as the percentage of TP relative to org. C, decreased to a minimum in 2009 when TP concentrations in the water were lowest (Fig. 8). The significant decrease over 30 years from 1988 to the latest sampling in 2018 was 17-32% for the three measures (Fig. 10).

As a measure of TP changes in sediment pools, we compared the particularly well reproduced measures in 1988 with 2018 (Table 4). This calculation showed a decline of the TP pool in the upper 5 cm sediment of 15 tons TP over the period. If we instead apply the 32% decline of TP per DW in the period 1988-2018 to the initial sediment TP pool in the upper 5 cm sediment, assuming a constant DW density, then the decrease of the sediment pool in the lake is 45 tons P. The two estimates correspond to annual mean reduction of the sediment pool in the lake at 0.5-1.5 tons. These rates are very small relative to annual mean TP inputs at 82 tons in the 1970s, 72 tons in 1985-87 and 6-7 tons P in recent years. The result agrees with the parallel course and magnitude of inputs and outputs from Lake Arresø over the 50 years period, while at the same time the mobile TP pool released to lake waters as summer progresses has decreased from about xx tons in the mid-1980s to about xx tons lately.

Not all P in the sediment is mobile P that may be released to the lake water and become available to phytoplankton and growth. Fractionation of TP pools in 1988 suggested that a high proportion (i.e. 48-77% of TP in the surface sediment including adsorbed and part of organic P) was mobile (COWIconsult 1988). TP fractionation data in 1993 suggested similar percentages of mobile P relative to TP of about 60% (Table Sx9). The small variation in the org C:TP ratio in the upper 10 cm over time suggests that organic matter was an important source of mobile TP to the water.

Discussion

Nutrient loading from urban and rural areas – The role of nutrient loading of Lake Arresø from urban relative to rural areas has undergone profound changes during the last fifty-sixty years. In the 1970-1980s, the high outlets from numerous domestic sewage treatment plants with traditional mechanical and biological purification totally dominated the large P-input to Lake Arresø, while N-input from rural areas slightly exceeded urban N-input. The restriction of the main domestic sewage treatment to two new plants of high purification efficiency outlets (including storm flow) brought outlets in 2018-2021 down to 3.5% for TP and 11.7% of former sewage outlets. This profound change took place despite residents and industries in cities (as person equivalents) increased by about 30% over the period (Danmarks Statistik).

Nutrient losses from cultivated fields has been less firmly regulated, but lack of direct measurements makes it difficult to quantify the losses. The estimated contributions from rural areas (with agriculture as the main land use and nutrient source) to the total inlet to Lake Arresø are presently about 90% for TN and 68% for TP when calculated as the difference between total inlets to Lake Arresø and urban outlets. Moreover, the estimated annual P loss from cultivated areas to Lake Arresø was 5.5 tons in 1986 (COWIconsult 1989). Assuming that the present loss is of the same order, it exceeds urban losses approximately 2-fold.

While TP loss from domestic sewage plants is close to the lower limits attainable with modern technology (i.e. about 0.15 mg TP L-1 in the outlet), this is by no means the case for TP and TN losses from cultivated fields. Since 2016, the permissible nutrient application of TP and TN from inorganic fertilizers and liquid manure on the fields has been allowed to increase by 20% following less restrict regulation and the protective 10 m wide buffer zones along streams have been cut to 2 m. Accordingly, extensive surface erosion of nutrient-rich soil particles from fields and stream banks are possible and easily observable (Fraser et al. 1994, Sand-Jensen et al. 2025). Nonetheless, critics in local media on improving water quality in Lake Arresø focuses on reducing urban TP losses even further, likely with limited effect, while ignoring the substantial nutrient losses from cultivated fields.

Annual nitrogen losses to the downstream N-limited estuary via the outlet from Lake Arresø’s have been reduced about 3-fold over the period from the 1970s (about 230 tons N) to the recent 2011-2021 (85 tons N) due to improved sewage treatment, construction of four new lakes in the catchment (removing N by denitrification) and lower N losses from cultivated fields. It may sound ironic, but construction of the four new shallow lakes with the goal of reducing P-inlets to Lake Arresø turned out to be without effect as lakes had no permanent sedimentation basins. Instead, denitrification in the new lakes contributed to lower N loading of the estuary which is critically threatened by N eutrophication (Geertz-Hansen et al. 1993, Pedersen and Borum 1996).

Nutrient dynamics in lake waters and biomass limitation – Dissolved phosphate concentrations in the lake outlet were apparently close to zero early in the period 1967-1976, while nitrate and ammonia were rather high pointing at an early P limitation of the phytoplankton if nutrient limitation was at play at all. During the following 10 years phosphate concentrations increased to high levels by 1973-1976. In the 1970-1980s, dissolved inorganic concentrations of both N and P during summer of high phytoplankton biomass rarely approached zero and when they did, inorganic N more frequently became low, while phosphate and TP increased as summer progressed. This situation is in agreement with the low TN:TP ratio of about 5 in the lake in the period and the low TN:TP ratio of about 3 in the high outlets of simple mechanical-biological sewage (Lønholdt 1973). Thus, conditions may have shifted from predominant P- to N- and back to P-limitation of the phytoplankton biomass from the mid-1960s to the early 2020s, considering that nutrient limitation occurred after all.

Over the subsequent fifty years to 2020, TP input decreased about 10-fold and TN decreased only two-fold, in-lake nutrient concentrations decreased and inorganic P and N more frequently fell close to zero during summer. The recent high TN:TP ratio between 20 and 30 suggests higher possibility of P limitation than N-limitation (Kolzau et al. 2014). Thus, the most likely nutrient limiting element of phytoplankton growth changed over the 50-60 years period as well as over the summer season with P being released and N denitrified in lake sediments.

However, limitation of phytoplankton primary production and biomass by other environmental resources (e.g. light or inorganic carbon) or co-limitation by light, inorganic carbon and nutrients during summer is also possible. Mean summer Secchi-depth in 1973 was 0.3 m (Andersen 1976), the euphotic zone (zeu = 1% of surface light) was about 1.0 m (Stefan et al. 1984) and the mixed zone (= mean water depth of 3.0 m) exceeded the euphotic zone 3-fold. Phytoplankton was in complete darkness at night and at very low light during most of the daytime (Andersen 1976). Nonetheless, annual phytoplankton primary production was very high (980 g C m-2, Andersen 1973) resembling that in other shallow hypereutrophic lakes (Table 21.4 in Kalff 2002) and the high chlorophyll biomass (about 300 mg Chl a L-1) could only be attained if phytoplankton cells were small, had high light absorption per unit chlorophyll and efficiently used low light (Agusti et al. 1994), while dark respiration rates must be very low (< 0.05 of light saturated photosynthesis; Andersen 1976, Henriksen 1977). Even restricted photosynthesis by inorganic carbon supply is possible as summer pH in Lake Arresø often was 9.5-10 and free CO2 only xx uM and the less usable HCO3- was xxx mM, when calculated from temperature, pH and alkalinities of 1.5-2.0 meq. L-1 (Andersen 1976, Rebsdorf 1973). Bioassay growth experiments with algal communities measuring biomass responses to N and P amendments and variable light and DIC conditions in combinations are required to evaluate such possible single or multiple limitations scenarios (Kolzau et al. 2014).

Sediment-water dynamics over time – Surface sediment and water were in close contact and decreased in concert over time with reduced nutrient input to Lake Arresø. The P concentrations in the upper 5 cm of surface sediments decreased significantly from 1986 to 2018 by a mean 32% or 45 tons P relative to the surface pool in 1988. Because the sediment density changes the estimated changes of the surface P pool over the period might only be 15 tons. Both measures are subject to considerable uncertainty due to sampling and measuring error, but the magnitude of sediment pools are within such narrow ranges. Thus, reductions of sediment pools in Lake Arresø of the order of 15-45 tons over 30 years are small compared to the total input over the period (xxx tons TP). An annual mean decline of 0.5-1.5 tons P would be difficult to catch by mass balances as it is within the uncertainty of input-output calculations.

The mobile sediment pool of TP and organic matter definitely declined from the 1970s to 2018-2021 according to the much smaller TP increase in the water column in the course of summer from initially xxx tons P to xxx tons P lately. Taking mid-1980 for comparison, the mobile sediment P pool released to the water column in 1986 was xxx tons higher than in 2018-2021, which is comparable to the estimated decline of P pools in the upper 5-cm sediment.

To conclude, we found that internal P loading has declined over the long-term period, but changes in mobile sediment pools are so small compared to the input to Lake Arresø that they do not represent a prominent delay to reduction of in-lake P concentrations and eutrophication. Earlier proposals to spend many million Euros to remove surface sediments across the 32 km2 of the most nutrient-rich lake bed and recent proposals to treat Lake Arresø with Phoslock to bind mobile P are not recommendable as external loading was and still remains the main regulator Lake Arresø’s eutrophication. Improvement of the ecological quality of Lake Arresø would require further reduction of the annual TP input to about 2 tons. This goal can only achieved by a marked decline of external nutrient sources primarily from cultivated fields and scattered housea in the catchment and secondarily by further decrease of outlets from sewage treatment plants and storm events.

The very frequent sediment resuspension can only be countered by building barriers in the lake basin to dampen waves and stimulating the dispersal and consolidation of submerged vegetation that previously dominated the lake during several thousand years. Realistically, we do not believe it will be possible to diminish P-input to the lake as shifting governments have been unwilling to effectively regulate P losses from cultivated areas, but it might find support experimenting with artificial barriers in restricted parts of the lake to evaluate the influence of sediment stabilization on submerged macrophyte colonization and, if successful, on better sediment particle retention and clearer waters. However, initiatives and money probably are better spend elsewhere on improving ecological quality in deeper lakes with more likely ecological improvement.

Litterature

Agusti S et al. 1994. Light harvesting among photosynthetic organisms. Functional Ecology 8: 273-279.

Andersen F Ø 1975. Arresø. MS. Thesis (in Danish). Freshwater Biological Laboratory, University of Copenhagen, Denmark

Andersen F Ø 1976. Primary production in a shallow water lake with special reference to a reed swamp. Oikos 27: 243-250.

Andersen J M 1974. Archiv für Hydrobiologie 74: 525-550.

Andersen J M 1976. An ignition method for determination of total  in lake sediments. Water Research 10: 323-331.

Arresø 2004. Status and Development 2003 (in Danish). Frederiksborg Amt, Teknik og Miljø. Landskabsafdelingen.

Björk S 1985. Lake restoration techniques and lake  pollution and recovery. Intern. Congr. European Water Poll. Cont. Assn, Rome: pp. 293-301.

Christensen C 1995. The Littorina transgression in Denmark. In: A. Fischer (ed.), Man in the Mesolithic. Oxbow Monographs, nr. 53: 15-32.

Danmarks Statistik. https://www.dst.dk.

Ellermann T. et al. 2010. Atmospheric deposition (in Danish). NOVANA. Danmarks Miljøundersøgelser, Aarhus Universitet. Faglig rapport fra DMU, nr. 761.

Fraser A I et al. 1994. The effect on rainfall intensity on soil erosion and particulate phosphorus transfer from arable fileds. Water Science and Technology 39: 41-45.

Freshwater Biological aboratory 1973. Limnological Methods (in Danish). Akademisk Forlag, Lyngby.

Geertz-Hansen O et al. 1993. Growth and grazing control of the marine macroalga Ulva lactuca in a eutrophic estuary. Aquatic Botany 46: 704-709.

Golterman H L 1969. Methods for chemical analyses of fresh waters. IBP Handbook No. 8. Blackwell, Oxford.

Heaney SI & W Davidson 1977. The determination of ferrous iron in natural waters with 2,2’ bipyridyl. Limnology Oceanography xx: 353-360.

Henriksen J 1977. Respirationens betydning for phytoplanktonets production Iavnadede eutrofe søer (Arresø 1976). MS Thesis, Freshwater Biological Laboratory, University of Copenhagen.

Hieltjes AHM & Lijklema L 1980. Fractionation of inorganic phosphates in calcareous sediments. Journal of Environmental Quality 9: 405-407.

Hovedstadsrådet 1975. Recipient pollution 1974. (Recipienters forurening 1974)

Hovedstadsrådet 1982. Studies of Lake Arresø 1976-1981 (in Danish). Work document from VKI and COWI. – Recipietundersøgelser af Arresø 1976-1981.

Hovedstadsrådet 1987. Lake Arresø’s water balance 1980, 1981 and 1982 (in Danish). Report no. 27. Det Danske Hedeselskab.

Hovedstadsrådet 1988. Phytoplankton i Arresø 1986. Recipientovervågning nr. 34. Rapport udarbejdet af Miljøbiologisk laboratorium

Hovedstadsrådet 1989. Restoration and future state of Lake Arresø (in Danish). Hovedstadsrådet, Copenhagen.  – Arresø 1985-1988. Tilsynsdata. Recepietovervågning nr. 45. Rapport udarbejdet af COWI.

Immers K H et al. 2015. Fighting internal phosporus loading: An evaluation of gradual Fe addition to a shallow peat lake. Ecological Engineering 33 : 78-89.

Jensen H S & Andersen F Ø 1992. Importance of temperature, nitrate and pH for phosphorus release from aerobic sediments of four shallow lakes. Limnology and Oceanography 37: 577-589.

Jensen H S et al. 1992. Iron: phosphorus ratio in surface sediment as an indicator of phosphate release from aerobic sediment in shallow lakes. Hydrobiologia 235/236: 731-743.

Jensen J P et al. 2006. An empirical model describing the seasonal dynamics of phosphorus in 16 shallow lakes after external loading reduction. Limnology & Oceanography 31: 791-800.

Jespersen A M & Christoffersen K Ethanol extraction.

Kalff J 1985 Limnology, Limnology. Prentice Hall, New Jersey.

Ketsev S et al. 2006. Factors controlling long-term P efflux from lake sediments: Explorative reactive transport. Chemical Ecology 234: 127-147.

Klein T 1989. Lakes in the good old days – on background status (in Danish). Vand og Miljø 5: 211-215.

Koroleff P 1970. Determination of total phosphorus in natural water by means og persulphate oxidation. An Interlab report No 3. Cons Int. l’exploration de la mer.

Landbrugspakken 2016. L 68 Fordlag til lov om jordbrugets anvendelse af gødning og plantedække. Folketinget, København, Danmark.

Lauridsen T et al. 2005. Lake Investigation. NOVANA Technical Instructions No 25 (in Danish). National Environmental Research Institute (NERI). Silkeborg, Denmark.

Lønholdt J 1973. Raw sewage’s concentrations of BI 5, N and P (in Danish). Stads- og Havneingeniøren 17: 1-7.

Mangerud J, Andersen S T, Berglund B & Donner J J 1974. Quaternary strategy of Norden, a proposal for terminology and classification. Boreas, nr. 3: 109-128.

Mertz E L 1924. Overview of Late and Postglacial changes of elevation in Danmark (in Danish). Danish Geological Surveys, 2. række, nr. 41.

Middelboe A L & Sand-Jensen K 2000. Long-term changes in macroalgal communities in a Danish estuary. Phycologia 39: 245-257.

Murphy J & Riley JP 1962. A modified single solution metho tfor determination of phosphate in natural waters. Anal Chem Acta 29: 21-26.

NOVA 2003. Vandmiljøplanens overvågningsprogram, https://www2.mst.dk

Olesen E M 1976. Lake Arresø (in Danish). MS Thesis, Freshwater Biological Laboratory, University of Copenhagen, Denmark.

Olsen S 1944. Danish Charophyta. Chorological, Ecological and biological investigations.. Det Kongelige Danske Videnskabernes Selskabs Biologiske Skrifter, 3: 1-240.

Osgood R P 1988. Lake mixing and internal phosphorus dynamics. Archiv für Hydrobiologie 113: 629-638.

Ostenfeld C E H 1928. Personal letters cited in Olesen (1976).

Pedersen M F & Borum J 1996. Nutrient control of algal growth in estuarine waters. Nutrient limitation and the importance of nitrogen. Marine Ecology Progress Series 142: 261-273.

Phillips et al. 1994.

Rebsdorf Aa 1972. The carbon dioxide system in freshwater (a booklet). Freshwater Biological Laboratory, University of Copenhagen.

Rydin E et al. 2000. Amounts of phosphorus inactivated by alum treatment in Wisconsin lakes. Limnology and Oceanogr. 45: 226-230.

Sand-Jensen K 2001. Lakes – a protected nature type (in Danish). Miljø- og Energiministeriet, Skov- og Naturstyrelsen. Gads Publishing Company.

Sand-Jensen K. 2014. Freshwater (in Danish). In: Nature in Denmark (ed. K Sand-Jensen). Gyldendal, Copenhagen.

Sand-Jensen K 2025. Arresø oplever øget eutrofiering. Vand & Jord 32: 14-17.

Stefan H G et al. 1983. Model of light penetration in aturbid lake . Water Resources Research 91): 107-120.

Solorzano L 1969. Determination of ammonia in natural waters by the phenyl hypochlorite method Limnology and Oceanography 14: 799-810.

Strickland JOH and Parsons TR 1968 A practical Handbook of Seawater Analysis. Fish Res Bd Can 167. Ottowa.

Søndergaard M et al. 1992. Phosphorus release from resuspended sediment in the shallow and wind exposed Lake Arresø. Hydrobiologia 228: 91-99.

Søndergaard M et al. 1993. Eight years of internal phosphorus loading and changes in the sediment phosphorus profile of Lake Søbygaard. Hydrobiologia 408/409: 145-152.

Søndergaard M et al. 2003. Role of sediment and internal loading of phosphorus in shallow lakes. Hydrobiologia 50 (1-3): 135-145.

Søndergaard et al. 2017. Hydrobiologia DOI 10.1007/s 10750-017-3110x

Søndergard M et al. 2012. Persistent internal phosphorus loading during summer in shallow eutrophic lakes. Hydrobiologia doi: 1007/s10950-D12-1o91-3.

Vandmiljøplan I (VMP 1, 1987), Vandmiljøplan II (VMP 2, 1988) and Vandmiljøplan III (VMP 3, 2004 ). Ministry of Environmental Protection, Copenhagen, Denmark.

Wesenberg-Lund C 1939. Brief information on lakes of North Zealand (in Danish). Frederiksborg Amts Historiske Samfund. Hillerød.

Welch EB & Dennis Clarke G 2008. Internal phosphorus loading in shallow lakes: Importance and control. Lake and Reservoir Management 21 (2): 209-217 DOI: 10.1082/07438140509354430.

Illustrations

Fig. 1. Map of Lake Arresø, outline of the catchment (stippled red) and location of the two main cities (Hillerød and Helsinge in grey). The four main stream-inlets (Ramløse Å, Pøleå, Æbelholt Å and Lyngby Å, blue lines) and forests (green) are also shown.

Figur S1. Chlorophyll concentration and Secchi-depth in Lake Arresø over time. Mean summer values (the lines) and ranges (coloured). Jonas: Benyt to paneler. Summer Secchi depth (cm).

Fig. 2. Annual input and output of TP and TN in Lake Arresø over 50 years.

Fig 3. Summer concentrations of TP, TN and TN: TP ratio during summer in Lake Arresø over 50 years. Mean values (the line) and ranges are shown.

Fig 4. Annual retention of TP and TN versus water retention time (WRT) in Lake Arresø over many years. Nutrient retention (= nutrient removal) is shown with positive value.

Figur S 2. Annual input (x) vs. annual outflow of TN (y) in the same year from Lake Arresø. The stippled line shows input = outflow (1:1 line). Inflow exceeds outflow in all years. The full line showing the relationship between measurements (y vs x, omitting the outlier in parenthesis) gradually approaches the 1:1 line at decreasing TN input suggesting linearly decreasing losses by denitrification as TN input declines.

Fig. 5. Annual input (above zero line) and removal (below zero line) of TP and TN in Lake Strødam and Lake Solbjerg and Lake Strødam in the years following their construction along the Stream Pøleå. .

Sedimenter

Figur 6. Sediment TP concentrations (per DW) with depth in different years.

Figur 7. The decrease of concentrations of TP (per DW), organic carbon (per DW) and TP (per organic carbon) in surface sediments (0-5 cm depth) over the study period. Significant differences between the first (1988) and later measurements are shown with horizontal lines and levels of significance (p < 0.05 *, p < 0.01 **, p < 0.001 ***).

Tabel 1.

Basic characteristics of Lake Arresø. For water retention time and alkalinity values are presented as mean (median, min-max).

| Catchment area (km2) | 215 |
| --- | --- |
| Cultivated area (km2) | 116 |
| Lake area (km2) | 39.9 |
| Mean depth (m) | 3.1 |
| Max depth (m) | 5.9 |
| Water retention time (years) | 2.4 (3.1, 1.3–5.9) |
| Alkalinity (meq. l-1)* | 2.47 (2.47, 1.75–3.35) |

* Alkalinity is calculated from summer values between 2000 and 2024

Tabel 2. Mean annual input of TP and TN to Lake Arresø and outlets from sewage treatment plants in 1970s, 1985-1987 and 2018-2021. Outlets during storm flow events in the two main cities are included in the sewage outlets in 2028-2021.

|  | 1970s |  | 1985–1987 |  | 2018–2021 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sources and proportions | ton year-1 |  |  |  |  |  |
|  | TP | TN | TP | TN | TP | TN |
| Input to Lake Arresø | 82 | 508 | 72 | 449 | 6.84 | 247 |
| % in 2018-21 of 85-87 |  |  |  |  | 9.5 | 55 |
| Output from water treatment plants and by stormflow |  |  | 62.6 | 199 | 2.22 | 23.3 |
| % input from urban areas of lake inlets |  |  | 87 | 44 | 32 | 9 |
| % urban input in 2018-21 of urban input in 1985-87 |  |  |  |  | 3.5 | 11.7 |

Tabel 3. TP-pools in sediment depth strata in Lake Arresø over the period. Estimates in tons P in soft sediments (31.6 km2) for the entire lake.

| Sediment depth |  | 1988 | 1997 | 2002 | 2009 | 2014 | 2018 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0–2 cm |  | 42.98 | 44.91 | 47.96 | 30.89 | 33.13 | 35.47 |
| 0–5 cm |  | 140.3 | 179.7 | 175.8 | 111.1 | 107.5 | 125.3 |
| 0–10 cm |  | 338 | 459 | 350 | 276 | 275 | 335 |
| 0–20 cm |  | 730 | 1111 | 749 | 607 | 599 | 781 |
| 0–40 cm |  | 1616 | 2719 | 1499 | 1206 | 1289 | 1716 |

Tabel S1. Mean chemical composition with depth in sediments (organic matter calcium, Fe and Fe:TP ratio).

| Sediment depth | Organic matter |  | Calcium |  | Fe |  | Fe:TP ratio |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mg (g DW)-1 |  |  |  |  |  |  |  |
|  | Mean | Range | Mean | Range | Mean | Range | Mean | Range |
| 0–2 | 372 | 277–450 | 165 | 152–177 | 9.7 | 6.8–13.7 | 7.2 | 4.0–10.87 |
| 2–5 | 346 | 237–420 | 147 | 136–157 | 10.3 | 7.1–13.3 | 9.3 | 3.7–17.1 |
| 5–10 | 300 | 180–360 | 132 | 113–150 | 11.1 | 6.9–14.4 | 11.6 | 3.9–21.8 |
| 10–20 | 270 | 180–340 | 156 | 132–180 | 12.1 | 9.8–15.8 | 13.6 | 9.6–24.7 |
| 20–30 | 231 | 184–314 | 134 | 111–156 | 13.5 | 9.5–15.7 | 16.3 | 10.3–22.8 |
| 30–40 | 220 | 188–263 | 126 | 125–127 | 14.1 | 11.6–17.7 | 20.8 | 15.0–25.6 |

Tabel S2. C:N, N:P and C:P ratios with depth in sediments over the years. Mean values (SD in parentheses).

| Sediment depth | 1988 | 1993 | 1997 | 1988 | 1997 | 1988 | 1993 | 1997 | 2002 | 2009 | 2014 | 2018 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C;N |  |  | N:P |  | C:P |  |  |  |  |  |  |
| 0–2 | 10.0 (0.65) | 8.1 (0.7) | 7.6 (0.4) | 7.83 (0.62) | 12.1 (0.22) | 78.3 (1.59) | 83 (24) | 98.0 (6.5) | 96.7 | 108.7 | 100.7 | 91.2 |
| 2–5 | 9.64 (0.41) | 7.4 (0.03) | 7.3 (0.3) | 8.14 (0.48) | 12.3 (0.79) | 78.4 (1.52) | 98 (21) | 94.5 (5.7) | 111.7 | 101.1 | 95.8 | 100.9 |
| 5–10 | 9.43 (1.05) | 7.5 (0.06) | 7.2 (0.3) | 9.18 (0.95) | 12.9 (1.8) | 86.2 (4.8) | 141 (8) | 94.3 (29.8) | 111.7 | 104.5 | 79.1 | 77.8 |
| 10–20 | 8.06 (0.23) | 7.4 (0.2) | 7.7 (0.2) | 10.5 (1.2) | 11.3 (0.77) | 93 (6) | 142 (6) | 90.6 (4.6) | 123.2 | 118.2 | 107.5 | 82.3 |
| 20–30 | 6.81 (0.51) | 7.7 (0.2) | 6.3 (0.5) | 12.1 (1.4) | 11.4 (1.1) | 107 (8) | 123 (5) | 76.3 (14.1) | 143.1 | 125.2 | 125.4 | 137.9 |
| 30–40 | 8.18 (0.06) | 7.9 (0.3) | 8.0 (0.2) | 16.0 (1.9) | 13.1 (3.6) | 138 (19) | 141 (38) | 105.5 (18.8) | 153.2 | 240.7 | 156.1 | 110.8 |

Tabel S4. Phosphorus separation with depth in sediment according to its association with different elements. Ad: loosely adsorbed. P-residual: total P – all other categories.

| Sediment depth | % P-Fe+Ad | % P-organic | % P-Ca | % P-Al | % P-residual |
| --- | --- | --- | --- | --- | --- |
| 0–2 | 24.39 | 26.81 | 35.40 | 3.00 | 10.41 |
| 2–5 | 13.18 | 30.26 | 38.80 | 3.10 | 14.65 |
| 5–10 | 15.39 | 30.44 | 37.90 | 1.00 | 15.27 |
| 10–20 | 12.85 | 35.18 | 30.90 | 2.80 | 18.27 |
| 20–30 | 13.35 | 28.52 | 35.80 | 5.40 | 16.93 |
| 30–40 | 12.51 | 37.14 | 30.50 | 1.30 | 18.55 |

Jeppesen E. et al. 2005. Freshwater Biology 50: 1747-1771.

End of this story

________________________________________________________________

Jeppesen E 1990.

Gibson et al. 2000. Lough Neagh Freshwater Biology 45:285-293.

Taking the largest town. Hillerød as an example, the sewage output contained 21 ton TP and 140 ton TN in the late 1970s while it had dropped to 0.96 ton P and 14 ton N in 2020-2023, though input to the water treatment plants had increased from 26,000 to 42,000 person equivalents (p.e.) in terms of the organic matter. Relative to p.e., the outputs of TP and TN in 2020-2023 compared with the 1970s are down to 1.9% and 5.8%, respectively.

Phosphorus purification in the two modern water treatment plants in the period 2018-2022 removed 97-98% of TP in the raw wastewater and brought outlets close to 0.15 mg TP L-1. TN purification was less efficient, but reached about 90% in the largest treatment plants in Hillerød.

The relationships between chlorophyll and TP or TN concentrations measured on the same days during summer likewise suggested the possibility of nutrient limitation of the phytoplankton when TP and TN concentrations during summer dropped below 0.3 mg P L-1 and TN dropped below 3 mg N L-1 (Fig. 9). Both linear regressions at low TP and TN concentrations are highly significant, but according to the coefficients of determination (R2), the variability of chlorophyll accounted for is higher for TP than TN.

The four shallow lakes were established to permanently reduce TP-loading and eutrophication of Lake Arresø, but it turned out that TN removal was much more significant for reducing the TN-loading of the downstream N-limited Roskilde Fjord.

The seasonal course of phosphate and dissolved nitrogen (nitrate plus ammonium) during the 50 years supported this pattern. During 1977, phosphate concentrations always were high (> 150 mg P m-3) and unlikely to limit the phytoplankton biomass, while this limitation may exist in 2022 (Fig. 8). Inorganic nitrogen concentrations dropped to low levels between June and October in 1977 and the period of possible limitation became longer in 2022 lasting from mid-April to mid-November and made co-limitation by both P and N likely.

Annual mean summer concentrations of TP, TN and chlorophyll in Lake Arresø water was divided into three main periods and the logarithmic change of concentrations was determined by linear regression over each period (Fig. 7). TP and TN concentrations decreased over the first period (1973-1990) and relatively more steeply over the second period (1990-2010), while they increased modestly over the third, most recent period (2010-2022). In contrast, chlorophyll concentration increased over the first period, while it followed the same the steep decline as TP and TN over the second period and their modest increase over the third period. The increase of chlorophyll during the first period was surprising and may be due to the shift from small chlorococcalean green algae to colonial cyanobacteria and cryptophytes that are easier to extract chlorophyll from and may have a smaller in vivo light absorbance per unit chlorophyll possibly contributing to a higher chlorophyll level. In fact, Secchi-transparency was lower in the 1970’ies than onwards (Fig. S1).

Species richness of submerged macrophytes remained low (up to 8 species, Table S1) and areal cover extremely low (x% of lake bed), confining rooted macrophytes to shallow waters at wind-protected sites (Olesen 1976). In contrast, the reed belt dominated by Phragmites australis (87%) had become larger and denser during the 20th..century reaching 2.76 km2 (7% of lake surface area) by 1973 (Andersen 1976).

Growth limitation of phytoplankton and macrophytes during fifty years in large, shallow Lake Arresø

Kaj Sand-Jensen1, Jonas Stage Sø2, Lars Baastrup-Spohr1 and Theis Kragh2

1Freshwater Biological Section, Department of Biology, University of Copenhagen, Universitetsparken 43floor, 2100 Copenhagen. 2

Introduction

Lake Arresø is an alkaline, shallow hypertrophic lake and the largest in Denmark (40 km2, Andersen 1976). It was a brackish plant-rich lake that became a plant-rich freshwater lake 4.000-5.000 years ago when the shallow connection to the saline Kattegat closed. During the 18-19th.centuries, the lake became eutrophic and during the 20th.century, it turned hypereutrophic and lost most of the submerged vegetation due to increasing nutrient inputs of domestic sewage from 60-70 thousand residents and nutrient losses from cultivated fields in the catchment (215 km2; COWIconsult 1989, Sand-Jensen et al. 2025).

Olesen (1976) documented that Lake Arresø already in the 1930s had become very turbid from dense phytoplankton blooms, while the narrow reed belts present early in the 1900s greatly expanded and became very dense due to the higher nutrient inputs (Andersen 1976, COWIconsult 1989). Very frequent particle resuspension from the upper 2 cm of the unconsolidated sediments due to the low mean water depth (3.0 m) relative to the large lake surface area and the high wind exposure contributed to further increasing light attenuation and lowering the Secchi-depth (COWIconsult 1989, Søndergaard et al. 1992).

Over the 50 years from the 1970s to the 2020s, the annual total phosphorus (TP) input to Lake Arresø decreased more than 10-fold from about 80 to 6-7 tons mainly due to restriction of sewage treatment from more than 30 small sewage treatment plants to only two new modern sewage treatment plants and simultaneously reductions of storm-water losses from the two largest cities (Sand-Jensen et al. 2025).

While the outlets from sewage treatment plants in the 1970s were rich in TP relative to TN (about 1:3, Lønholdt 1973), the modern treatment plants in the early 2020s have much lower outlets overall and in particularly efficient TP-purification (about 98%) changing TP:TN ratios in the outlets to 1:14. Nutrient losses from cultivated fields are presently responsible for about 90% of TN and 50% of TP inputs to Lake Arresø (Sand-Jensen et al. 2025). Over 50 years these changes have reduced nutrient concentrations in the lake and elevated TN:TP ratios from about 5:1 to above 20:1, thereby, increasing the likelihood of phytoplankton biomass limitation by nutrients and from N rather than P.

Rate limitation of the phytoplankton production and yield limitation and yield limitation of the phytoplankton biomass are subject to complex regulations by nutrients, light availability and inorganic carbon supply (Ref). The periods of potential nutrient limitation with undetectable or very low dissolved inorganic concentrations of phosphate, nitrate and ammonia likely have become more frequent and longer. Moreover, phytoplankton biomasses and chlorophyll concentrations may have declined as a consequence of stronger nutrient limitation over time.

The ongoing extensive resuspension of sediment particles and high phytoplankton biomass may maintain a narrow euphotic zone relative to the turbulent mixed zone and restrict the upper limit of phytoplankton biomass by light limitation (Ref). In addition, high biomass and production rates of the phytoplankton in the 1970-1990s (Andersen 1976, Olesen 1976) generated very high pH levels (9.5-10.0) and only traces of free CO2 conducive to rate limitation of phytoplankton photosynthesis (Kragh and Sand-Jensen 2018, Hammer et al. 2019). Depending on the extent of pH reductions and phytoplankton adaptations to use CO2 and HCO3- as carbon sources, the likelihood and extent of rate limitation of phytoplankton may have changed over the 50 years.

We present the development of chlorophyll concentrations and phytoplankton biomasses during the summers over the 50-years period. In order to evaluate the possible multiple limitations of phytoplankton, we first examined the likelihood of nutrient limitations from the inorganic P and N concentrations, the TN:TP ratios and the chlorophyll concentrations or biomasses relationships to TP and TN. Then, we determined the ratio of the euphotic zone to the total mixed zone and the mean light availability in the mixed zone in relation to the phytoplankton chlorophyll and biomass levels including the possible effect of suspended sediment particles. Subsequently, we determined the mean availability of CO2 and HCO3- to the phytoplankton during the summer period. Finally, we discuss possible zooplankton grazing effects in addition to the environmental restrictions on phytoplankton biomass development.

Materials and methods

Lake Arresø’s dynamics of environmental conditions, phytoplankton and zooplankton have been studied by several authors and agencies in the 1970-1980s (Andersen 1976; Hovedstadsrådet 1975, 1982, 1988; Olesen 1976, COWIconsult 1989 and references in them). An early 10-years long series of monthly measurements of inorganic nutrient concentrations (1967-1967) by Copenhagen’s Water Supply in the outlet from Lake was even available (Olesen 1976). From the late 1980s to the early 2000s, detailed measurements following a standard program were conducted by the Frederiksborg County and, thereafter, by The Agency of Environmental Protection (Lauritsen et al. xxxx). The measuring program followed the standards of the Danish National Monitoring Program (NOVANA), which has been running since 1989. It includes mid-lake sampling at monthly intervals during winter and bimonthly during summer for the main physical (i.e. temperature and Secchi depth), biological (chlorophyll, phytoplankton and zooplankton composition and biomass) and chemical variables. Individual phytoplankton and zooplankton were counted, their dimensions measured on a representative number of individuals and their volume or weight estimated. Chemical measures include TP and TN analysed by spectrophotometry after persulphate digestion (Koroleff 1969, Solorzano 1969). Inorganic phosphate, ammonia and nitrate were measured according to several authors (Murphy and Riley 1962, Strickland and Parsons 1968, Solorzano 1969). Chlorophyll was initially determined by acetone extraction on GF/C filters (Golterman 1969) and later replaced by ethanol extraction (Jespersen and Christoffersen xxxx). Total alkalinity was measured by acidimetric titration (Rebsdorf 1973), pH by a Radiometer combination electrode and ionic strength from specific conductivity and conversion tables in Rebsdorf (1973). Total inorganic carbon (DIC) and the proportions of CO2, HCO3- and CO32- were at ambient temperature and ionic strength were calculated according to xxxx.

OMGIVELSES VARIABLE

Næringssalte

Fig 1. TP, TN og TN/TP i søen normal skala over hele året og årrækken.

Figur 2. Uorg P og Uorg N i 1977 og 2022 (mærket 10) OK

Figur 3. Uorg P, nitrat og ammonium gennem årene (indsat begrænsende niveauer) 2503 Kb (mærket 7) OK

Lysforhold

Fig. 4. Secchi-transparency og fotisk zone (y, tv. meter), fotisk zones andel af den omrørte vandsøjle (y, th.).

Table Secchi depth (y) vs chlorophyll and vs. suspended solids and vs both over the years.

Uorganisk kulstof

Fig. 5. Alkalinitet, Reduktion af alkalinitet fra vinter/forår til sensommer som mål for nettoproduktionen i søen.

Fig. 6. pH og fri CO2, evt. in-outflux af CO2

Algebiomasse og sammensætning

Fig. 7 Chlorophyll

Table Chl vs TP, vs TN and versus TP and TN

Fig. 8. Algebiomassen opdelt I grupper og som kulstof.

Fig. 4. Algeklorofyl, og Secchidybde i Arresø over hele året.

NB! Måske skal figur 3 fra forrige artikel behandles her og dermed fortidens vegetationsdynamik og nutidens vandplanter.  (figur og tabel).

Fig. 3. Algebiomassens størrelse og fordeling mellem algegrupper (stolpediagram stakkes ovenpå).

Figur 4. Uorg P og Uorg N i 1977 og 2022  (mærket 10) OK

Figur 5. Uorg P, nitrat og ammonium gennem årene (indsat begrænsende niveauer) 2503 Kb (mærket 7) OK

Figur 6. pH, alkalinitet og CO2 i vandet i sommersæsonen over årene.Pil evt nogle år ud og vis detaljerne der ved høj og levere næringsindhold.

Supplement Secchidybde i relation til algeklorofyl og suspenderet stof.

Middel Secchi-depth og range during summer season. År for år

1938	1956-57    1976	                   1980ies             1990ies (summer means)

30/1	20;15-35/3	30;11-38/10   40.4(0,29-0,49)       40.0 (0.30-0.43)

JONAS: inkl. alle årene i 1980, 90, 2010 og 2020’erne i Arresø.

Tabel Suppl 1, statistik. Sammenhængen mellem Secchi-dybde og klorofyl og evt suspenderet stof, hvis det findes data for nogle år.

Figur 3. Algebiomassens størrelse og fordeling mellem algegrupper.

Figur 4, Fig. 8. Uorg P og Uorg N i 1977 og 2022  (mærket 10) OK

Figur 5. Uorg P, nitrat og ammonium gennem årene (indsat begrænsende niveauer) 2503 Kb (mærket 7) OK

Figur 6. pH, alkalinitet og CO2 i vandet i sommersæsonen over årene.Pil evt nogle år ud og vis detaljerne der ved høj og levere næringsindhold.

Dissolved concentrations of inorganic phosphate, nitrate and ammonium are other proxies for the likelihood of phytoplankton nutrient limitation. Phosphate concentrations below 5 ug P L-1 and nitrate and ammonium concentration below 25 ug N L-1 may indicate phytoplankton biomass limitation (shaded grey area in Fig. 7). From year 2000, phosphate concentrations dropped more frequently below 5 ug P L-1 suggesting more frequent nutrient limitation, except for the most recent year. Low nitrate and ammonium concentrations (< 25 ug N L-1) frequently occurred through the summer in all years. Thus, limitation of phytoplankton during summer by low inorganic N concentrations is possible in all years, while phosphate limitation gradually became more likely over the years in agreement with the profound increase of the TN/TP ratio.

(((Input of P and N from open land rural areas - The loss of TP and TN loss from cultivated areas in the catchment during the last 40-50 years is poorly known due to very few direct measurements. Legislation regarding the annual permissible application of inorganic fertilizer and liquid manure on the cultivated fields has changed over time from lower permissible application during the 1990s to higher application after 2016. Moreover, legislation in 2011 demanded 10 m wide uncultivated buffer strips along major streams, but this legislation was abandoned in 2016.

A recent model evaluation (2016-2018) based on standard values for the contribution of annual nutrient loads to Lake Arresø estimated a 55% contribution of TP and 35 of TN from rural areas (including water treatment plants) and 45% contribution of TP and 65% of TN from farmland and forests. After 2018, better control of stormflow events and improved tertiary sewage treatment from the two large water treatment plants in the two largest cities have further reduced nutrient losses from the rural areas.

The mean annual TP input was 80.5 tons from domestic sewage to Lake Arresø in 1973-1979, but 13.4 tons TP in in 1991-1994 and only 1.5 tons TP in 2020- 2023, excluding stormflow, events and 2.24 tons TP when including them. These values suggest that that despite many more inhabitants and higher industrial activity in the major cities of the catchment, the rural areas now contribute about 40% to the extensively reduced annual TP input to Lake Arresø at about 6.8 tons pointing at cultivated farmland as a larger contributor even of TP.  Improved purification has brought TP concentrations down to only 0.15 mg P L-1 in the outflow from the two major sewage treatment plants.

Taking the largest town. Hillerød as an example, the sewage output contained 21 ton TP and 140 ton TN in the late 1970s while it had dropped to 0.96 ton P and 14 ton N in 2020-2023, though input to the water treatment plants had increased from 26,000 to 42,000 person equivalents (p.e.) in terms of the organic matter. Relative to p.e., the outputs of TP and TN in 2020-2023 compared with the 1970s are down to 1.9% and 5.8%, respectively.

Removal of P and N in Lake Arresø and new lakes – Nitrogen was removed in Lake Arresø every year over the last fifty years from the 1970s to the 2020s (Fig. 4). Because the nitrogen pool in the upper sediments did not increase, but in fact decreased by about 16% from 1988-2016 along with the reduction of organic matter, this removal primarily represents a net loss by denitrification. Over the years, the removal decreased along with the reduced input (Fig. 4 and 9). In contrast, annual input and output of TP changed almost in concert over the years, though long water retention time retained some TP in the lake, while short water retention time released some TP.

During the first two years after constructing the shallow Lake Strødam and Lake Sobjerg they removed a high proportion of TP and TN input via the Pølå stream (Figure 11). The loss of TP during passage is probably due to particle net sedimentation and P-adsorption in the submerged terrestrial soils. After the two initial years, however, TP retention became minimal as a balance between particle sedimentation-resuspension and P-adsorption and release was attained. The loss of TN may also be due to particle net sedimentation and to denitrification in the sediments. After the two intial years, nitrogen loss continued in Lake Solbjerg likely due to ongoing denitrification.

Assuming that the annual loss per surface area in the four constructed shallow lakes in the catchment to Lake Arresø was the same as the annual mean loss in Lake Solbjerg at 14.5 g TN m-2 in year 3, 4 and 5 after its construction, the combined annual loss was 21.5 ton N from the four lakes (surface area of 147.8 ha). The mean annual N loss by passage of Lake Arresø in the recent 10 years 2012-2021 was 95.4 ton N. Thus, the sum of removal was 117 ton N from all five lakes in the catchment. Compared to the mean annual TN output of 84 ton from Lake Arresø in the same 10 years, the combined TN removal by denitrification in the five lakes was 39% higher than the output of TN from the catchment via the outflow from Lake Arresø. If denitrification had not kane place, TN output from the Lake Arresø catchment would have been 139% higher. The four shallow lakes were established to permanently reduce TP-loading and eutrophication of Lake Arresø, but it turned out that TN removal was much more significant for reducing the TN-loading of the downstream N-limited Roskilde Fjord.))

Late glacial and postglacial development

From freshwater lake to saline fjord and back to freshwater lake – After the Weichel’s Ice Age ended 11,700 years ago, a Late glacial and Postglacial period followed (10,000-9,300 years before present, BP) with an oceanic water level reaching about 35 m below the present one, and Arresø as a relatively deep freshwater lake (Mangerud et al. 1974, COWIconsult 1989). Over the following thousands of years, several meters thick freshwater and marine gyttje deposited in the lake basin. During the marine transgression period (9,300-6,000 years BP), the oceanic water level increased to 5 m above the present sea level (Christensen 1995) turning Arresø into a low saline fjord with a shallow connection to the saline Kattegat (Figure 1). Gradual greater land rising relative to the sea level by about 5 m during the last 6,000 years (Mertz 1924), and deposition of marine sand carried by coastal currents formed a land bar across the shallow 8-10 km wide strip between Arresø and Kattegat gradually closing their connection permanently between 3,000-2,000 years BP (COWIconsult 1988). Excavation of the Arresø Canal in the mid-18th.century formed a regulated lake surface level and a new outlet to the saline Roskilde Fjord, and made Lake Arresø the 41 km2 large and shallow (mean 3.1 m) freshwater lake it remains today (Sand-Jensen 2003, Table 1, Figure 2).

Studies of plant macrofossils and pollen in a 200-cm deep sediment core revealed a vegetation development divided into five periods over the recent 3,000 years (Klein 1989, Figure 3). The oldest Period 1 covers the gradual change from a low-saline fjord to the freshwater lake. Thus, the oldest sediments in Period 1 contain shells of Mytilus edulis and small, thin shells of Cerastoderma edule, but no Oestrea edulis shells as in the neighboring more saline waters in Kattegat and Roskilde Fjord (Figure 1). This shows the low saline character of Arresø (COWIconsult 1988). The remains of aquatic macrophytes are species that can grow both in low-saline and fresh water (i.e., Stuckenia pectinata, Ruppia sp. and Zannichellia sp.). The presence of Persicaria amphibia reveals the freshwater influence that was prominent close to the inlets of freshwater streams to the lake. During Period 1, charophytes gradually became more important, probably reflecting decreasing salinity over time. Period 2 has few fossils perhaps due to a shifting salinity caused by periodic input of saline water from Kattegat across the shallow land bar north of the lake making species survival difficult before the connection to Kattegat closed permanently. In Period 3, Arresø is a clear-water, vegetation-rich freshwater lake with dense stands of charophytes, Zannichellia sp., Myriophyllum spicatum, Stuckenia pectinata, Potamogeton berchtoldii and P. pusillus. In Period 4, Arresø remained a clear-water lake with a rich Potamogeton-Stuckenia vegetation, but with fewer charophytes. Up to Period 5, dominance of submerged macrophytes and transparent waters prevailed (Figure 3). The change to Period 5, likely happened during the early 18th century in connection with greater human influence by cultivation in the catchment and changing lake water levels stimulating leaching of nutrients and humic substances from the nearby wetlands (COWIconsult 1988). As a result, submerged macrophyte coverage almost disappeared, while nutrient demanding emergent reed belts expanded (primarily Phragmites australis, secondarily Typha angustifolia and Scirpus lacustris, Andersen 1976). Biologists observed gradual expansion of the reed belts between 1910 and 1940 (Ostenfeld 1928 and Wesenberg-Lund 1939 cited in Olsen 1976).